Release Notes                                       Page 1 of 24

CTOS TCPIP 2.2

December 1993

CONTENTS                                                    PAGE

Product Interdependencies...................................  2

Restrictions and Known Limitations..........................  4

Product Improvements........................................  7

Migration Guidelines........................................ 18

Support..................................................... 19

Product Information Updates................................. 23

Release Notes                                        Page 2 of 24

PRODUCT INTERDEPENDENCIES

One of the following operating system levels is required to run

CTOS TCPIP 2.2:

BTOS II 3.0, or higher, for workstations

CTOSXE II 3.2, or higher, for XE530 shared resource

processor systems

CTOSXE II 3.0, or higher, for XE520 shared resource processor

systems

IDMSS 4.0, or higher, for port expansion modules

In addition, CTOS TCPIP requires the following products:

BNet II 2.0, or higher for Cluster Access

X.25 Gateway 9.0, or higher, for operation over an X.25

network

Release Notes                                        Page 3 of 24

PRODUCT INTERDEPENDENCIES  (continued)

Context Manager II 3.0, or higher, for TELNET and FTP

OFIS Mail 1.2, or higher, for SMTP

Indexed Sequential Access Method II (ISAM) 1.1.6, or higher,

for FTP

CTOS PC Emulator 4.1, or higher, for running the ClusterShare

version of the CTOS Socket Library

MS DOS 4, or higher, for running the ClusterShare version of

the CTOS Socket Library

Release Notes                                        Page 4 of 24

RESTRICTIONS AND KNOWN LIMITATIONS

ISAM files are not copied correctly during FTP transfers.

If it is necessary to transfer ISAM files between two CTOS

systems, the data file should be transferred to the target

system in type 'Image' and then ISAM Reorganize should be

invoked on the target system to generate the index file.  This

procedure should be adopted for ISAM, ISAM II, and ISAM III.

The TCPIP FTP Client cannot be invoked directly from the

Context Manager.  It is necessary to execute the Executive

from the Context Manager first and then invoke the TCPIP

FTP Client.

Release Notes                                        Page 5 of 24

RESTRICTIONS AND KNOWN LIMITATIONS (Continued)

If Telnet is receiving large amounts of data in one context

of the Context Manager, applications in another context

cannot be loaded.  To avoid this problem, ensure applications

are executing prior to receiving the Telnet data.

On occassion, after copying a file successfully, the FTP

Client will return an error message.

Due to an Ofismail bug, attchments received via SMTP on a

password protected disk are scrambled.  A future release of

Ofismail will correct this problem.

Telnet does not operate under Administrator Cluster View.

Release Notes                                        Page 6 of 24

RESTRICTIONS AND KNOWN LIMITATIONS (Continued)

If a file is to be transferred to a remote system via FTP,

it is first copied to a temporary file in directory

[sys]<tcpip.ftp>.  If the FTP server cannot copy the file,

it is not deleted.

A file with a badly formed specification e.g. 'ba[', will

cause the FTP command 'DLIST' or 'FLIST' to return

incomplete information.

IP should be deinstalled whenever TCP is deinstalled

otherwise IMSP will crash shortly after TCP is reinstalled.

Release Notes                                        Page 7 of 24

PRODUCT IMPROVEMENTS

FTP Client

New Interactive Client supporting background and foreground

operation and many new commands

CommandForm Client for background operation

FTP Local Server

Queue Manager dependence replaced by internal queues

FTP log transactions and performance statistics

FTP log transaction trace

FTP log control utility

Release Notes                                        Page 8 of 24

PRODUCT IMPROVEMENTS (Continued)

FTP Remote Server

FTP support for additional commands (MSend, Mretrieve,

CD, etc.)

Improved access security

Support for anonymous remote FTP users

FTP server remotehelp support

FTP server implementation of full range of FTP commands

FTP server implementation of multiple commands during

single login

Release Notes                                        Page 9 of 24

PRODUCT IMPROVEMENTS (Continued)

Improved performance for FTP file transfers

SMTP

Accepts inbound mail addressed to IP addresses

Inbound mail with no CTOS center address now addressed

to local center

New SMTP status information from OFIS Mail R8.2 Interactive

Mail Manager (IMM)

Provides SMTP support for encoded and unencoded mail

attachments

Transparent CTOS to CTOS attachments

Choice of UNIX uuencode, normal, or FTP retrieval methods

Release Notes                                        Page 10 of 24

PRODUCT IMPROVEMENTS (Continued)

SMTP support for continuation

Support for mail interoperation with CC:Mail

IP

Dualnetwork interface IP address  (X.25 and Ethernet)

Support of subnets via Netmask

Routing (Gateway) capabilities between X.25 and Ethernet

network interfaces

Configurable default link type

IP broadcast capability from UDP

Release Notes                                        Page 11 of 24

PRODUCT IMPROVEMENTS (Continued)

Preconfigurable ARP table option

NGI

X.25 NGI support for default X.25 address

API

Sockets are compiler independent

Both CTOS C and Microsoft C can be used

Pascal is better supported

Object modules can now be linked with the socket library

by customers of third parties  (without customers purchasing

a Clibrary license)

Release Notes                                        Page 12 of 24

PRODUCT IMPROVEMENTS (Continued)

New functions for:

UDP broadcast

Netmask management

Local IP address management

Hardware

Support for XE530 Ethernet

New hardware

Supported on a sharedresource processor  (XE520530)

Release Notes                                        Page 13 of 24

PRODUCT IMPROVEMENTS (Continued)

Miscellaneous

FTP, TELNET, and ping support for unconfigured hosts

Protocol Manager (PM) now replaces Data Link Manager (DLM)

Software Distribution (SWD) supported

3.5inch floppy disks supported

Release Notes                                        Page 14 of 24

PRODUCT IMPROVEMENTS (Continued)

IP now installs correctly even if a previous installation

failed.

Error 2450 no longer occurs wnen running X.25 NGI server.

The Date and time display for TCP utilities is now correct.

NGI now fully recovers following an incorrect X.25 DISC.

TCPIP SMTP logfile password now remains concealed.

Simultaneous crashes of all network servers no longer occur.

TCPIP Operations Manager now displays meaningful values.

Release Notes                                        Page 15 of 24

PRODUCT IMPROVEMENTS (Continued)

When using TCPIP over X.25 over switched ISDN connections,

the link now opens correctly

Large connection handle values no longer cause server to

ERC 23.

Frequent failures to connect to host no longer occur.

Sporadic crashes in IP, due to invalid options, no

longer occur.

VAX VMS host no longer reports an error 425 when working with

the FTP server.

When running X.25 NGI server, error 2450 no longer occurs when

Max. X.25 VCs' parameter is greater than 10.

Release Notes                                        Page 16 of 24

PRODUCT IMPROVEMENTS (Continued)

SMTP mail messages are now correctly deleted after successful

delivery

Server running TCPIP and X.25 no longer crashes with erc 12

when file transfer fails.

When performing file transfer from UNIX to CTOS, FTP now asks

for password before opening CTOS .user file.

Reception of IP broadcast frames no longer causes erc 80

crash.

SMTP manager installation no longer fails if the attachment

directory is password protected.

Release Notes                                        Page 17 of 24

PRODUCT IMPROVEMENTS (Continued)

Socket library no longer crashes with error 58976 following

successful usage

Error 60235 no longer occurs when sending UDP datagram

smaller than 1042 bytes

Error 60239 no longer occurs when using more than 20 sockets

with an equal number of ports

When using TCPIP over TELNET, and host timeout occurs, CTOS

will now close the connection.

When using TCPIP over X.25 and running the VI editor, the

NEXT PAGE key no longer causes IP to crash.

Release Notes                                        Page 18 of 24

PRODUCT IMPROVEMENTS (Continued)

CTOS TCPIP s2.2.05 or higher is required if a socket application needs to

increase the number of sockets and the application is compiling with BTOS C.

If BTOS C 1.1 is being used, recompile socklim.c with the large memory

model and with the option D__BTOSC__.  If BTOS C 1.2 or higher is being

used, the option D__BTOSC__ is no longer needed.

FTP append no longer erases most of the original document.

Release Notes                                        Page 19 of 24

MIGRATION GUIDELINES

Following are restrictions and recommendations for using CTOS

TCPIP with other Unisys software products:

An existing TCP configuration file (tcpip.params) must

be rebuilt using the R2.2 TCPIP Configuration Manager.

Release Notes                                        Page 20 of 24

SUPPORT

TCPIP 2.2 is a warranted Unisys software product.

Unisys warrants that TCPIP 2.2, in its unaltered form,

will substantially conform to Unisys current published functional

specifications when used in Unisys supported configurations.  The

customer is responsible for reporting suspected deviations using

the User Communication Form (UCF) process within ninety (90) days

following delivery.  The product is neither field repairable nor

field modifiable.  Corrections to reported deviations are

provided to the user via updated code files.

This standard warranty is not a substitute for Unisys Service

Agreement offerings.Release Notes                                        Page 21 of 24

SUPPORT (continued)

Support Category

TCPIP 2.2 is fully supported by Unisys.

Support services for TCPIP 2.2 are offered under the

SURETY (United States) or A la Carte (International) support

programs and are consistent with the support services offered for

the entire CTOS Hardware and Software family of products.

Release Notes                                        Page 22 of 24

SUPPORT (continued)

Unisys SureNet is an electronic bulletin board service, providing

customers in the United States access to technical information

seven days a week, 24 hours a day.  SureNet Technical Information

Services are available at no additional cost to customers with a

current Unisys service agreement or software license and include

access to system alerts, customer technical bulletins and online

UCF entry. Registration is required.  SureNet Support Services are

available on an annual subscription basis.  For more information,

contact Unisys Direct at 18004481424, prompt 5.  You may

register for Technical Information Services andor Support

Services either online by dialing 18008288796 (8 data bits,

1 stop bit, no parity, asynchronous, VT100 emulation) or by

calling Unisys Direct.

Release Notes                                        Page 23 of 24

SUPPORT (continued)

If you need further assistance or information regarding support

services, please contact your Unisys representative or your local

Unisys office.

Support Discontinuance

Unisys supports the current release level and one level back.

Prior levels are not supported.

The CTOS TCPIP software is in Support Category A and Warranty

Class 1.  It is fully supported and enhanced, and it is

warranted for 90 days to conform to Unisys published

specifications.

Release Notes                                        Page 24 of 24

PRODUCT INFORMATION UPDATES

CTOS TCPIP Administration Guide, Page 422

The last parameter of the Install SMTP Manager command has been

changed from Mail Relay Parameters to Relay Domains.