CLUSTER COMMUNICATIONS SPECIFICATION

GENERAL

The general protocol of the operating system dictates that the master workstation poll the cluster workstations, and that the master workstation and the cluster workstations exchange messages. The protocol requires that every workstation on the cluster communucations line (or all workstations in a minicluster) have a unique workstation identification number.

The master workstation initiates all communications; a cluster workstation recognizes messages intended for it by the workstation identification number. The cluster workstation picks a workstation identification number by:

o	minitoring the cluster communications line to find an

unused number

o	taking an unused number

o	monitoring the cluster communications line again to see

if it has collided with another workstation that may

have picked the same number

If a collision occurs, both workstations wait a random time interval before restarting the search for another identicication number.

The workstation identification number ranges from 1 to 15 (the largest number of cluster workstations allowed on a single cluster communications line). Identification numbers are not the same as user numbers. User numbers are assigned to cluster workstations by the operating system of the master workstation and are different for every workstation in a cluster. Identification numbers are determined by the cluster workstations and are duplicated on the different cluster communications lines of a cluster.

PROTOCOL DEFINITION

The linklevel cluster communications protocol is a subset of the American National Standars for Advanced Data Communications Control Procedures (ADCCP), as defined in ANSI X3.661979, published by the American National Standards Institute, Inc., using UN (Unbalanced Normal) Class of Procedure, Twoway Alternate transmission with optional functions 4, 5 and 6 (Unnumbered Information, Initialization and Unnumbered Polling) used for booting and dumping. The bootstrapdump protocol is illustrated in Figure 1. Symbols are as follows:

Symbol	Function

SNRM	Set normal response mode

RIM	Request initialization mode

SIM	Set initialization mode

XID	Identification frame*

UP	Unnumbered poll

UI	Unnumbered data frame

RD	Request disconnect

DISC	Disconnect

UA	Unnumbered acknowledge

UI'	Unnumber data frame (with

termination data)

* XID sent by the cluster workstation has an Ifield five bytes in length, containing the parameters desired by the workstation. (Refer to the XID CommandResponse IField Definition subsection, below.)

Upon an XID of the format given in the XID CommandResponse IField Definition subsection, below, the master workstation sends an XID of the same format, indicating the desired parameters, and expects to receive a UA in response. (This also occurs on a protocol startup not involving a boot or dump; for example, a local file system.)

DUMP           Master Workstation:           Bootstrap ROM:

SNRM >

SNRM >

SNRM >

<	RIM

SIM  >

<	XID

XID  >

<	UA

UP   >

<	UI

*   *   *

UP   >

<	UI

UP   >

<	RD

DISC >

<	UA

BOOTSTRAP         Master Workstation:           Bootstrap ROM:

SNRM >

SNRM >

SNRM >

<	RIM

SIM  >

<	XID

XID  >

<	UA

UP   >

<	UA

UI   >

*   *   *

<	UA

UI   >

<	UA

UI'  >

Figure 1. Cluster Communications BootstrapDump Protocol.

When SYSGENed to run the line at 307 kilobaud to support B22 and B21 workstations, a master will not send XID; if the workstation does not receive an XID from the master, the workstation will assume the line is to be run at 307 kilobaud. Note that the RIM and XID that start off the bootstrapdump sequence are always sent at 307 kilobaud; if XID negotiation so indicates, the parties then switch to the speed selected by the master workstation.

XID COMMANDRESPONSE IFIELD DEFINITION

Byte No.   Function

0	OS type

1	Software revision level

2	Maximum line speed allowed

0 = 1200 baud

1 = 2400 baud

2 = 4800 baud

3 = 9600 baud

4 = 19.2 kilobaud

5 = 56 kilobaud

6 = 153 kilobaud

7 = 307 kilobaud

8 = 615 kilobaud

9 = 1.23 megabaud

10 = 1.54 megabaud

11 = 1.84 megabaud

3	Window size (default = 1)

4	Higher level protocol version:

0 = Current cluster, big reads

split at cluster workstation

(default)

1 = Current cluster, big reads

split at master workstation

2 = BTNet low interconnect

3 = BTNet medium interconnect

4 = BTNet high interconnect

5	Hardware type

0 = B22

1 = B21

2 = B25

3 = XE500

6	Error indicator