Most of the assembly language programs released on Micro Cornucopuia disks

are set up for the M80 assembler. If you've been wondering what the .MAC

extension is for, those are M80 source code programs. Many of the people

that got this disk did so to assemble the M80 source files without spending

$150 for M80. It can be done.

For the most part .MAC files can easily be translated to .AZM with very few

changes. Unfortunately M80 is as extremely versatile assembler which when

used to its full advantage contains some pseudo-ops and directives that Z80MR

does not understand. I will reveal the most common of these and provide

some help on getting around them in this file.

First things first.........................................................

Rename the file to have a .AZM extension. Edit the file. Near the beginning

of the file you will see the pseudo op that tells M80 what mnemonics you

will be using. M80 can handle both Z80 and 8080 mnemonics. You can even

switch in the middle of the program. Two pseudo-ops .Z80 and .8080 tell the

assembler which mnemonics to recognize. If there is a .Z80 there delete

it. If there is a .8080 there you will need to run the program through an

8080 to Z80 translator (like XLATE2.COM on disk M13).

At this point what I usually do is run the program through the assembler.

The assembler is very good at finding errors, so I let it tell me what it

doesn't like. You can run the program through the assembler without

generating any output files (see AZM-COM.DOC) and still get the error

report on the console. A ^P before running the assembler will dump to the

printer as well. Now you can go back into the file and fix the syntax where

the assembler generated errors.

Dual mnemonic files........................................................

XLATE2 is a great translator on 8080 only files. It also will only translate

the 8080 mnemonics in a file that has both. Unfortunately there are a couple

of Z80 mnemonics that are the same as 8080 mnemonics and will be dutifully

translated by XLATE2. The problem is that the mnemonics are the same but the

instructions they represent are not. So the file will assemble OK but will

run with unpredictable results. An example.

JP	is a valid Z80 mnemonic. It is also widely used since

it is an unconditional jump (JMP in 8080)

JP 	is also a valid 8080 mnemonic. It is the jump on parity

instruction.

So a Z80 JP is translated as if it was the 8080 JP which will make it

JP	P in the output file. Bad news. This in itself is enough to make

it too undependable to use on dual mnemonic files. To avoid the grief you

can do one of two things:

Translate the 8080 sections by hand if they are small enough.

Split the file into smaller files where the mnemonics change.

Run the 8080 sections through the translator. You can let the

assembler rejoin the files using the *INCLUDE statement or

use pip or your editor to rejoin and assemble the whole mess.

What I'd really like to see is someone rework XLATE2 so that it would stop

translating when it saw .Z80 (and also comment the pseudo-op out) and

start again when it saw .8080 (and comment it out as well). Also some of

the other minor syntax changes could be fixed as well. Yes XLATE2 can be

assembled with Z80MR (it is a .MAC file). After removing the .Z80 at the

beginning of the file the only error I got was for the MOD operation on the

version number (MOD not supported by this assembler). My solution, just

enter the version number there. Having the assembler figure out the ascii

just wasn't that important.

Macros.....................................................................

The M80 macros go through Z80MR pretty well for the most part. There are

some syntax changes to be made.

Precede the parameters with # signs (including locals).

Also there are some macro pseudo-ops that are not supported (also not widely

used). If some one writes Z80MR macros to substitute for these (and a MOD

operator) please send it in and we'll include it in the macro library that

we are developing for this assembler.

.phase and .dephase........................................................

These are used to make the assembler generate label addresses for some other

part of memory than where the file will be loaded. The file PHASE.DOC

offers some alternative ways of doing this.

#################################################