README file for PALASM 4 version 1.5

----------------------------------------------------------------

Unless otherwise noted the following items apply to both PAL

and MACH parts.

1. Simulation:

-- Please read section 2.2 in the 1.5 release notes.

-- Be aware that DATA I/O Unisite only allows 12 input

changes per vector, even at startup.

-- If state names are used in the simulation section and

minimize changes the original polarity, then check clashes

may occur.

-- Set and reset are not exchanged for registered pins whose

polarity is changed by MINIMIZE. Hence, check clashes  in

simulation may occur when using the "Best For Device",

"Active-High" or "Active Low"  options in the Logic

Synthesis submenu.

-- In-circuit loading of MACH internal registers and observa-

tion of stored values is not supported at this time.

-- Simulation for the MACH parts may be inaccurate if there

are any floating pins in the design file and no PLC file

has been generated by the FITR prior to invoking the

simulator.  This is due to the pin could be in any bank

and so there is no knowing what functional equations

should be applied to it.

-- The following parts use the "old simulator":

16RA8 20RA10 22IP6 23S8 32R16 64R32.

For such parts, the SIMULATION option under RUN in

the menu should NOT be selected. If SIMULATION is

selected, the run will terminate after PARSE. The BOTH

option under RUN should be selected for these parts.

2. DOS 5.0 & EXTENDED memory PALASM

To run the extended memory version of PALASM 4 version 1.5

under DOS 5.0 a memory manager is required:

- PALASM 4 is not a WINDOWS program ( a separate memory

manager is required ) and should be run as a

standalone program.

- HIMEM.SYS is not sufficient by itself; another memory

manager is still required. When no other memory manager

used, BIOS errors are issued by HIMEM's DOS extender

routines.

After installation is complete the CONFIG.SYS file should be

changed to configure the system properly. The following two

sample configurations provide MS-DOS compatibility when

used with the Microsoft EMM386  and Quarterdeck Office Systems

QEMM memory managers.  They have been tested on a system

configured with most utilities and device drivers in \sys

directory.

I. CONFIG.SYS File Using Microsoft EMM386 & HIMEM.SYS:

DEVICE = C:\SYS\SETVER.EXE

DEVICE= C:\SYS\HIMEM.SYS

DOS=high,umb

device=c:\sys\emm386.exe 2048 m9

break = on

buffers = 10

files =35

device=c:\sys\smartdrv.sys 2048 1024

devicehigh=c:\sys\ansi.sys

shellc:\command.com /e:1512 /p

II. CONFIG.SYS File Using Quarterdeck QEMM

( version 5.13 or higher ) & Microsoft HIMEM.SYS:

DEVICE = C:\SYS\SETVER.EXE

DEVICE= C:\QEMM386.SYS RAM MAPS=12 HANDLES=96 NS

break = on

buffers = 10

files =35

device=c:\loadhi.sys /r:2 c:\sys\smartdrv.sys 2048 1024

device=c:\loadhi.sys /r:1 c:\sys\ansi.sys

shellc:\command.com /e:1512 /p

3. KNOWN PROBLEMS

--- If you have problems with the software, please check Appendix 2

in the 1.5 release notes.  The problem may already be known and

a work-around documented.

4. PAIRING PROBLEMS ( MACH PARTS ONLY ):

--- BPP does not create SET or RESET equations for node

equations that are input-paired to PIN equations. It is

mandatory to specify SET/RESET on both of the input-paired

equations.

--- A registered/latched pin or node may be output paired by

BPP with a combinatorial node or pin. This is illegal in

MACH devices. Such pairings are ignored by the Fitter with

an appropriate warning. However, simulation timing may be

affected by the resulting implementation.

--- BPP may create ( illegal ) duplicate equations when

either the node or the pin of a PAIR is preplaced and the

other is floating.

5. MINIMIZE may run out of memory in two ways:

---  64K segment problem. In the new MINIMIZE program

furnished with this release, the 64 segment problem

does not typically happen unless an equation has many

product terms ( i.e 25 to 30 ) and each product term

has only one or two unique signal names.  A sparse

Karnaugh map results, which probably can't be

minimized much further anyhow, therefore manually

"gate split" the offending equation.

---  If the extended memory MINIMIZE executable runs out of

memory, then the user should select the "FAST

MINIMIZATION" option in the PALASM 4 "LOGIC SYNTHESIS"

menu. Note, that an "extended" out of memory usually

happens only when options DT, gate splitting, and

"Best For Device" ( polarity ) are all set simultaneously.

Therefore, an alternate strategy to the Fast Minimization

option is to reduce the number of MINIMIZATION options

selected. Typically, "Best For Device" is the most

memory intensive of the options.

--- When choosing "Best For Device" be aware that if

Minimize chooses the opposite polarity than the

original, then when a reset is realized a following

check on the outputs  will be the opposite value.

6. Parse will give a GP error outside the menu when it detects

an unusual character (i.e umlauts) rather than give an "un-

expected character" error, which it does inside the menu.

7. Backannotation ( MACH only ) of truncated names

( > 14 characters ) does not work.  Backannotation of

unused pins and nodes can lead to duplicate pin numbers in

the design file -- which causes a failure in PARSE.

8. GROUPS:

- Group names cannot be used within the Simulation section

to specify values for a collection of signals;

- GROUP statements in the Declaration section must appear

after all PIN or NODE statements.

9.  PARSE may run out of memory on rare occasions even in

extended memory.  Two designs of this type involved very

complex CASE statements and many STRINGS. The only work-

around when this occurs in extended memory is to substitute

the strings manually throughout the design instead of

allowing the parse program to do it for you.

10. GP errors, ZMALLOC and runtime errors produced by PALASM 4

programs should be forwarded with your name, phone number

and .PDS file to AMD Corporate Applications ( 800-222-9323 ).

Many previously reported errors of this type have been fixed

in this release.

11. For a 16V8 PAL device, the REG attribute when specified in

the PIN statement for pin number 16 causes PALXPT to give an

error ( X3670 ).  Workaround: For pin number 16, remove REG

from the PIN statement and use := in the equation for the

pin.

12. If a file is too large to view under the VIEW option of the

menu, then it may be viewed using the editor by selecting

the "other file" option under EDITOR and supplying the

appropriate file name.  Since MACH230 report files are

larger than for other MACH parts, the "too large to view"

problem is more likely to occur now than in previous

releases. Note under the SETUP option the user substitute

any ascii editor for the PALASM 4 default editor, ED.

Unfortunately, the DOS 5.0 built-in editor, QEDIT, is not

compatible with PALASM 4.

13. If you are having diffucties with the extended version running

on notebooks and laptops (especially TI's), then read the section

1.3 in the 1.5 release notes.